Assessment No.  1

Risk Assessment Form

| Completed by and role: Stewart Neve, Fleet Commissioning Manger / Roy Tanner, Workshop Manager | Initial assessment date: 03/05/2019 |
| --- | --- |
| Location of the risk: Mercedes Sprinter 519 DCA (RX19XTG) |  |
| Task / Hazard being Assessed: Complete vehicle - Fleet |  |

| STEP 1 |  |  | STEP 2 | STEP 3 | STEP 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REF NO. | ACTIVITY | HAZARD | PERSON(S) AFFECTED AND HOW | CURRENT CONTROL MEASURES | RISK RATING L x C = R |  |  |
| 1 | Maintenance | Engine running | Fleet staff | - Keep clear of moving parts / viscous fan - Hot components in the engine bay - Be aware of excess heat - Avoid fumes from the exhaust / use extractor fan - Do not select “drive” option when the engine is running - Ensure handbrake is applied and wheels chocked - Keep long hair tied back - No loose clothing to be worn near running engine - Ensure the bonnet is correctly propped open, using manufacturers recommended / supplied stays etc. - Be cautious when working under the bonnet with high winds - Ensure all people are stood clear of the vehicle when starting the engine – Handbrake applied & out of gear - Wear suitable PPE | 2 | 4 | 8 |
| 2 | Maintenance | Fluids | Fleet staff | - Do not cross contaminate fluids - Use correctly labelled containers - Only use fluids approved by vehicle manufacturer - Vehicle fluids may be very hot - Avoid direct skin contact - Keep ignition sources away from fluids and fumes - Clean up all spillages - Wear PPE & use skin protection – barrier cream etc. | 2 | 2 | 4 |
| 3 | Operating the vehicle | Vehicle controls | Fleet Staff | - Gear change paddles – familiarisation - Column stalk gear lever – familiarisation - New dash layout – familiarisation - Only staff holding a C1 category on their licence to drive vehicle | 1 | 2 | 2 |
| 4 | External charger | Electric shock | Fleet staff | - Do not use with wet hands - Do not use with damaged cables - Isolate / disconnected from the mains supply prior to working on system | 1 | 4 | 4 |
| 5 | Accessing saloon heater | Burns / fumes / personal injury | Fleet staff | - Avoid contact with exhaust pipe which could be hot - Avoid fumes which may be present from operation of the saloon heater - Use exhaust gas extractor when heater is running - Ensure the heater has shut down prior to being worked on. - Allow heater to cool before working on the system | 2 | 3 | 6 |
| 6 | Wheel removal & refitting | Personal injury | Fleet staff | - Be aware of the weight of the wheel - Use lifting aids to assist removal and refitting of wheels when possible - Ensure all road wheels are secure to the vehicle using manufactures wheel nut settings before driving - Use wheel nut indicators when wheel nuts are torqued up - Use protective handware / gloves, especially if the tyre is damaged - Wear suitable PPE & skin care cream | 2 | 3 | 6 |
| 7 | Tail lift operation & repairs | Personal injury | Fleet staff | - Be aware of the weight of some components which will be heavy - Tail lift system under hydraulic pressure - Possible figure trap on hand rail - Only trained staff to work on the system. - Ensure the lift platform is on the ground or supported prior to disconnecting the hydraulic system (System not under load) - Ensure 3rd parties keep their distance when lift is being operated. - Move weights individually when carrying out the LOLOR test - Wear suitable PPE | 2 | 3 | 6 |
| 8 | Emergency lighting & siren system | Personal injury | Fleet staff | - Avoid direct eye contact when emergency lighting is switched on - Avoid prolong use of siren system – operate with mute system when possible - Use ear defenders when siren is being worked on / tested - Ensure system is powered down / disconnected prior to working on the system - Ensure working at heights training is completed when working on lights high up - Ensure ladders etc have been regularly tested | 1 | 3 | 3 |
| 9 | Saloon area servicing | Personal injury | Fleet staff | - Refer to manual handling procedure when lifting medical equipment from brackets - Isolate and report all piped gas leaks - Open and close cupboard panels with figures away from the edge due to figure trap hazard - Ensure all equipment is correctly secured to it bracket when not being worked on - Only qualified engineers to work on the piped gas system / equipment - Use hand rails when entering and leaving the saloon area - Take caution when reaching out of site areas due to potential sharps injuries | 2 | 3 | 6 |
| 10 | Working on air conditioning system | Personal injury | Fleet staff | - Isolate and repair / report all gas system leaks - Only trained staff to work on the system - Wear suitable PPE & skin care cream | 1 | 3 | 3 |
| 11 | Using saloon inverter | Electric shock | Fleet staff | - Isolate system when being worked on - Do not touch live wires - Do not insert objects into sockets - Switch system off when not required for use | 1 | 4 | 4 |
| 12 | Road test | Personal injury | Fleet staff | - Follow road sign warnings - Always use safety devices (Seatbelts etc) - Drive following highway code / Road transport law. - Do not place feet on dash if a passenger where air bags are positioned - Always remain seated - Risk of RTC - Obey all warning devices - Do not attempt to open any vehicle doors whilst vehicle is in motion - Ensure when possible a banksman is used when reversing - Do not attempt to adjust steering column whilst driving - Be aware of the vehicle’s height restrictions - Driver to hold correct category on there driving licence | 2 | 3 | 6 |
| 13 | Door & Cupboard openings | Figure trap | Fleet Staff | - Doors to only be opened and closed using appropriate handles - Obey all signage advising of how to close the doors | 1 | 3 | 3 |
| 14 | Use of side facing seats on DCA’s | Side facing seats | Staff, patients and passengers In the event of an accident, seatbelts on these side facing seats may help to prevent the wearer being thrown around the vehicle or from being ejected, but in a frontal crash they can increase injury risk by subjecting vulnerable parts of the body to higher loads than seatbelts used on forward facing seats | - British or European standards and be marked accordingly with either the ‘e’, ‘E’ or BS ‘Kitemark’. - The seatbelt anchorage points are designed so that they will be capable of withstanding the high forces of an impact - Seatbelts and anchorages are professionally installed by qualified persons. - Where there is an ability to turn side facing seats to face forward then this position should be adopted during the carrying any staff, patients or passengers. - Where side facing seats cannot be positioned to face forward they can be used if no other seats are available in the saloon. | 2 | 2 | 4 |

| Date Signature | 1st Review date 26/05/2020 Signature S.Neve | 2nd Review date Signature | 3rd Review date Signature | 4th Review date Signature | 5th Review date Signature | 6th Review date Signature |
| --- | --- | --- | --- | --- | --- | --- |

- Risk scoring:	The risk score will be based upon the consequence of a risk and the likelihood of it being realised.  The Trust uses the risk scoring methodology and matrix previously published by the National Patient Safety Agency;

- Scoring the Consequence: Consequence must be scored using the Table of Consequences as a guide:

| Table of Consequences |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Domain: | Consequence Score and Descriptor |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 |
|  | Negligible | Minor | Moderate | Major | Catastrophic |
| Injury or harm Physical or Psychological | Minimal injury requiring no / minimal intervention or treatment No Time off work required | Minor injury or illness requiring intervention Requiring time off work < 4 days Increase in length of care by 1-3 | Moderate injury requiring intervention Requiring time off work of 4-14 days Increase in length of care by 4-14 days RIDDOR / agency reportable incident | Major injury leading to long-term incapacity/disability Requiring time off work for >14 days | Incident leading to fatality Multiple permanent injuries or irreversible health effects |
| Quality of Patient Experience / Outcome | Unsatisfactory patient experience not directly related to the delivery of clinical care | Readily resolvable unsatisfactory patient experience directly related to clinical care. | Mismanagement of patient care with short term affects <7 days | Mismanagement of care with long term affects >7 days | Totally unsatisfactory patient outcome or experience including never events. |
| Statutory | Coroners verdict of natural causes, accidental death or open No or minimal impact of statutory guidance | Coroners verdict of misadventure Breech of statutory legislation | Police investigation Prosecution resulting in fine >£50K Issue of statutory notice | Coroners verdict of neglect/system neglect Prosecution resulting in a fine >£500K | Coroners verdict of unlawful killing Criminal prosecution or imprisonment of a Director/Executive (Inc. Corporate Manslaughter) |
| Business / Finance & Service Continuity | Minor loss of non-critical service Financial loss of <£10K | Service loss in a number of non-critical areas <6 hours Financial loss £10-50K | Service loss of any critical area Service loss of non- critical areas >6 hours Financial loss £50-500K | Extended loss of essential service in more than one critical area Financial loss of £500k to £1m | Loss of multiple essential services in critical areas Financial loss of >£1m |
| Potential for patient complaint or Litigation / Claim | Unlikely to cause complaint, litigation or claim | Complaint possible Litigation unlikely Claim(s) <£10k | Complaint expected Litigation possible but not certain Claim(s) £10-100k | Multiple complaints / Ombudsmen inquiry Litigation expected Claim(s) £100-£1m | High profile complaint(s) with national interest Multiple claims or high value single claim .£1m |
| Staffing and Competence | Short-term low staffing level that temporarily reduces patient care/service quality <1day Concerns about skill mix / competency | On-going low staffing level that reduces patient care/service quality Minor error(s) due to levels of competency (individual or team) | On-going problems with levels of staffing that result in late delivery of key objective/service Moderate error(s) due to levels of competency (individual or team) | Uncertain delivery of key objectives / service due to lack of staff Major error(s) due to levels of competency (individual or team) | Non-delivery of key objectives / service due to lack/loss of staff Critical error(s) due to levels of competency (individual or team) |
| Reputation or Adverse publicity | Rumours/loss of moral within the Trust Local media 1 day e.g. inside pages or limited report | Local media <7 days’ coverage e.g. front page, headline Regulator concern | National Media <3 days’ coverage Regulator action | National media >3 days’ coverage Local MP concern Questions in the House | Full public enquiry Public investigation by regulator |
| Compliance Inspection / Audit | Non-significant / temporary lapses in compliance / targets | Minor non-compliance with standards / targets Minor recommendations from report | Significant non-compliance with standards/targets Challenging report | Low rating Enforcement action Critical report | Loss of accreditation / registration Prosecution Severely critical report |

3. Scoring the Likelihood: Likelihood must be scored using the Table of Likelihood as a guide:

| Description | 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Almost Certain |
| --- | --- | --- | --- | --- | --- |
| Frequency (How often might it / does it occur) | This will probably never happen/recur Not expected to occur for years | Do not expect it to happen/recur but it is possible it may do so Expected to occur at least annually | Might happen or recur occasionally Expected to occur at least monthly | Will probably happen/recur, but it is not a persisting issue/circumstances Expected to occur at least weekly | Will undoubtedly happen/recur, possibly frequently Expected to occur at least daily |
| Probability | Less than 10% | 11 – 30% | 31 – 70 % | 71 - 90% | > 90% |

- Risk Score and Grading: Once the Consequence and Likelihood have been determined, the over-all risk score can be measured using the Risk Score Matrix and should follow a linear pathway:

| 1 Rare | 2 Unlikely | 3 Possible | 4 Likely | 5 Almost certain |
| --- | --- | --- | --- | --- |
| 5 | 10 | 15 | 20 | 25 |
| 4 | 8 | 12 | 16 | 20 |
| 3 | 6 | 9 | 12 | 15 |
| 2 | 4 | 6 | 8 | 10 |
| 1 | 2 | 3 | 4 |  |

- For grading risks: the scores obtained from the risk matrix are assigned grades as follows:

|  | 15 to 25 Extreme Risk |
| --- | --- |
|  | 8-12 High Risk |
|  | 4-6 Moderate Risk |
|  | 1 to 3 Low Risk |

- Risk grading: Supports the Trust with setting its risk appetite, with the ‘Risk Rating - Actions Table’ used to define the guidance on the documentation/ registration of the risk on DATIX, the urgency of action to mitigate the risk and clarifies ownership, reporting and oversight:

| Score | Risk Grade | Action * | Risk Owner ** | Operational Monitoring *** | Organisation Oversight **** | Escalation Route | Board Assurance |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-3 | Low | Risk entered onto DATIX | Principle Risk Lead | Group (described in each terms of reference) | Management Group / Committee / Board (described in each terms of reference) | Described in each: Group/ Committee/ Board terms of reference | Quality and Patient Safety Committee Finance & Investment Committee Workforce and Wellbeing Committee Audit Committee |
| 4-6 | Moderate |  |  |  |  |  |  |
| 8-12 | High |  |  |  |  |  |  |
| 15-25 | Extreme |  |  |  |  |  |  |

- Additional control measures may well be implemented following the registration of the risk onto the Trusts risk register. Subsequently risk scores may change. If this happens add the control measure to the risk assessment when implemented and update risk scores.

References:

- Health and Safety Policy

- Risk Management policy

- Risk Management procedure